An energy company is considering an investment in wind-power production. To determine whether this investment is economically feasible, the company seeks to estimate the expected revenue of a wind-power producer operating in a competitive spot market.

Consider a day-ahead spot market with five electricity producers. Three producers generate electricity from conventional sources, such as coal, whereas the remaining two rely on solar and wind power, respectively. The market is cleared once each day for 24 hourly trading intervals of the following day. For each interval, every producer submits a bid that specifies the electricity quantity it is willing to supply (MWh), together with the minimum price it is willing to accept ($/MWh). The market operator then applies a merit-order clearing mechanism to determine both the market-clearing price and the dispatch schedule, subject to the requirement that total electricity demand be satisfied. If a producer’s bid is accepted, the producer is obligated to supply the nominated quantity and is remunerated at the market-clearing price. If the bid is not accepted, the producer receives no revenue for that interval.

The energy company intends to develop a model to estimate the expected revenue of the wind-power producer on any future day by determining the hourly bid quantities and prices under uncertainty. On this basis, the model can subsequently be extended to examine outcomes across a range of short-term and long-term scenarios, thereby supporting an assessment of the investment’s economic viability.

Please formalize this problem as an agent-based model, and specify the key components, including time scales, agents, state variables, control variables, processes, objectives, constants, uncertain parameters and stochastic variables. At this stage, the model should be limited to simulating a single day of market operation.